User Manual 
ROOTDataLogger I & II					Version March 2025 (ahp1)

Caution:

Always use a 12 V battery. Depending on the type, the battery must be removed for transport—gel or liquid batteries may leak if stored improperly.

- The currently installed AGM battery should be leak-proof.

Before data collection:

Charge the battery (ON/OFF switch to OFF.)
Delete old files on the SD card.

Data Collection:

- Insert the SD card.

- Connect the sensors:  
– Connect the load cell to the LOAD socket (6).

– Connect the draw-wire sensor to the CABLE socket (5).

- Select the correct load cell (2kN/20kN) using switch (8).

Switching the load cell automatically performs a TARE.

- Set the ON/OFF switch (4) to ON.

(When switching on and off, a new .csv is generated in each case).

- Note the exact time of the measurement.

- Set the BT-MON/LOG switch (1)

  - When the green LED (3) flashes, the measured values are being saved directly to the SD card.

  - When the blue LED (2) flashes, the measurement data can be read via Bluetooth (using RootDataLogger I or II, password: 1234) or displayed on a laptop via the USB socket (7).

  - If no LED is flashing, check whether the SD card has been inserted correctly.

⚠️ Important: 
When the blue LED is flashing, data is not being saved to the SD card.

Apps (only Android devices for now (not iPhone compatible))

App for Android:

Serial Bluetooth by Kai Morich

App for Windows

Serial port monitor (free version)– not yet tested

2

1

2

3

4

5

6

7

5

6

1

3

2

Technical data

At 12.2 volts, an average of 140 mA (1.16 mA idle), 
i.e., a fully charged battery with, for example, 6 Ah, lasts theoretically for 
approx. 40 hours (6 Ah/0.14A = 42.9 h).

Note that the lead-acid battery (AGM) is fully charged at a terminal voltage of 12.8 V (100 %) and discharged at a terminal voltage of approx. 11.8 V (0 %).

Calibration:

LEANE – DBBE 200 kg: factor 10050 – calibrated with a 20 kg and 40 kg load – May 2022

ME-Systeme - 5000 kg_ (factor 660 – measured with a 40 kg and 80 kg load – May 2022

Parts List

| No° | Article | Description | Seller |
| --- | --- | --- | --- |
| 1 | Basetech Outdoor IP67 | Case 460 x 360 x 175 mm | conrad.ch |
| 2 | Neutrik NC5FD-LX-B | 5-Pol socket | conrad.ch |
| 3 | Neutrik NC5MX | 5- Pol plug | conrad.ch |
| 1 | Neutrik NAUSB-W-B | USB plug | conrad.ch |
| 1 | Neutrik SCDP-0CON | Seal for sockets | conrad.ch |
| 1 | TC-R13-208B-02 12 V/DC 20 A | Switch (green LED) | conrad.ch |
| 1 | TC-R13-208A-02 250 V/AC 10 A | Switch (no LED) | conrad.ch |
| 1 | Arduino Uno | Microcontroller | conrad.ch |
| 1 | TC-9927152 | HC-05 Bluetooth module (Android only) | conrad.ch |
| 1 | Joy-it SEN-HX711-20 | HX 711 - Amplifier board for the load cell | conrad.ch |
| 1 | Adafruit Assembled Data Logging shield | SD Shield Logger for Arduino Uno | conrad.ch |
| 1 | SD Card | 32 GB | conrad.ch |
| 1 | Motobatterie YTX7A-BS Okay | Battery 12 V / 6Ah | Landi.ch |
| 1 | Kemo Spannungswandler (3 - 15 V/DC 1.5 A) | Voltage transformer/stabilizer | conrad.ch |
| 1 | 110x80x70mm IP67 | Transparent plastic housing Arduino | bastelgarage.ch |